SCANTEST 1.00

TABLE OF CONTENTS

WHAT IS SCANTEST? . . . . . . . . . . . . . . . . . . . . . . . . .    2

HOW TO USE SCANTEST . . . . . . . . . . . . . . . . . . . . . . . .    4

WHAT ARE SCAN CODES?  . . . . . . . . . . . . . . . . . . . . . . .    8

DECIMAL KEYBOARD SCAN CODES . . . . . . . . . . . . . . . . . . .   10

A WORD FROM THE SPONSOR . . . . . . . . . . . . . . . . . . . . . .   11

OTHER PROGRAMS  . . . . . . . . . . . . . . . . . . . . . . . . . .   12

WARRANTY  . . . . . . . . . . . . . . . . . . . . . . . . . . . . .   13

REVISION HISTORY  . . . . . . . . . . . . . . . . . . . . . . . . .   14

SCANTEST 1.00            Page 1 of 14

WHAT IS SCANTEST?

�����������������

SCANTEST is a small utility designed to help you see what is going on

with your keyboard.  It inserts itself between the keyboard and the

computer, and shows you the Scan Codes that are being sent from the

keyboard to the computer.

Most people have a little bit of difficulty understanding Scan Codes,

and especially understanding the difference between Scan Codes and ASCII

characters.  The section below on page 8, "WHAT ARE SCAN CODES?",

explains this in a little more detail.  For now, let me just say that

what you normally see appear on the screen when you type something from

the keyboard is an ASCII character, not a Scan Code.  What a particular

key does when you press it is determined by the software inside the

computer, and in reality has NOTHING at all to do with the letters you

have imprinted on the plastic "keys" on your keyboard.

There are dozens of standard keyboard layouts available that you can

purchase (mostly designed for languages other than United States

English).  The insides of the keyboards are all the same - the only

difference is what is imprinted on the plastic "keys".  You can even

pull the plastic "keys" off the keyboard, rearrange them all you want,

and it won't change anything.  In addition to changing the plastic

"key", you also have to change the software inside the computer.

The main purpose of this program is to use as a companion

troubleshooting tool with one of my other programs, called SCANCODE.

SCANCODE is a utility that can "type" for you automatically.  However,

SCANCODE is different from other "typing" utilities because it "types"

Scan Codes.  Most other "typing" utilities don't really "type" at all,

but rather stuff the keyboard buffer with ASCII characters.  SCANCODE

can "type" into many more programs than an ASCII-stuffing utility, and

can even "type" into Windows.

If you play around with SCANTEST, you will discover that sometimes

keyboards send some VERY strange things to the computer.  There is a lot

of history behind it all, created by IBM when it converted from the

original 84-key PC keyboard to the 101-key AT keyboard (which is what

all new computers still use).  IBM made some very poor design decisions

(in my opinion) when they converted to the AT keyboard, and we have had

to live with and work around those poor decisions ever since.  I rant

extensively about this in the documentation for SCANCODE, so I won't do

it again here.

SCANTEST 1.00            Page 2 of 14

You also can use SCANTEST to give you some idea what is happening if you

have a very unusual keyboard, like the ones that usually come on laptop

computers.  Laptop keyboards nearly always eliminate certain keys that

are on a regular keyboard, combine multiple keys into one, and add extra

keys of their own (such as a key to display the status of the battery).

Some programs can get confused by what the laptop keyboard sends,

because it is sometimes VERY different from what a regular keyboard

would send under the same circumstances.  SCANTEST may be a useful

troubleshooting tool if you find yourself in such a situation.

SCANTEST 1.00            Page 3 of 14

HOW TO USE SCANTEST

�������������������

To use SCANTEST, you simply type the following:

SCANTEST [Filename]

where [Filename] is optional.  [Filename] can be a simple file name

(like SCANTEST.OUT), a file name that includes a path (like

C:\TESTS\SCANTEST.OUT), or a Device name (like PRN or LPT1).

When you do this, SCANTEST inserts itself between the keyboard and the

computer, and shows you, on the screen, what <keys> are being pressed

and released.  If you entered a [Filename], SCANTEST sends the

information to both the screen and [Filename] at the same time, so you

can have a hard copy to look at later.

When you are ready to quit SCANTEST and have the computer go back to

normal again, you press and release the <Escape> key.

In addition to showing you the decimal Scan Code for a particular key,

SCANCODE also gives you a Description of the <key>.  The Description is

what the key is on a standard United States QWERTY keyboard.  If your

keyboard is not really a QWERTY, the Description may not match what is

imprinted on your plastic "key" (but the decimal Scan Code will ALWAYS

be correct).

Let's go through a sample session using SCANTEST.  Lets say you type

SCANTEST at a DOS prompt, and then type the following keystrokes on the

keyboard:

<Shift>

<A>

<Enter>

<CapsLock>

<Pause/Break>

<Ctrl>-<Pause/Break>

<PrintScreen/SysReq>

<Ctrl>-<Alt>-<Del>

<Escape>

NOTE:  This assumes you have an AT keyboard, which is what all computers

since the IBM PC/AT have used.  It also assumes that you are

running "real" DOS, and not running from a DOS box underneath

Windows.  The test does not work the same from a DOS box.

SCANTEST 1.00            Page 4 of 14

This is what the output of SCANCODE would look like:

Codes in {brackets} are the keys being released.

Press <Esc> to Quit.

SCAN CODES      DESCRIPTION (US KEYBOARD)

-----------------  -------------------------

42                Shift

{170}              {Shift}

-----------------  -------------------------

30                A

{158}              {A}

-----------------  -------------------------

28                Enter

{156}              {Enter}

-----------------  -------------------------

58                CapsLock

{186}              {CapsLock}

-----------------  -------------------------

225- 29- 69        Pause

{225-157-197}      {Pause}

-----------------  -------------------------

29                LeftControl

224- 70            Break

{224-198}          {Break}

{157}              {LeftControl}

-----------------  -------------------------

224- 42            FakeShift

224- 55            PrintScreen

{224-183}          {PrintScreen}

{224-170}          {FakeShift}

-----------------  -------------------------

29                LeftControl

56                LeftAlt

83                Delete

{157}              {LeftControl}

{184}              {LeftAlt}

{211}              {Delete}

-----------------  -------------------------

1                Escape

{129}              {Escape}

-----------------  -------------------------

SCANTEST 1.00            Page 5 of 14

As you can see, this shows you both the decimal Scan Code of each <key>

pressed and released, as well as a Description (the Description is only

accurate if you are using a United States QWERTY keyboard, however).

Scan Codes and Descriptions that are NOT in {brackets} are the <keys>

pressed.  When a <key> is released, the Scan Code(s) and Descriptions

are put in {brackets} and indented by several spaces.  I know this is

not the prettiest output screen you've ever seen, but it's still quite

useful.

When SCANTEST is running, it intercepts all of the keys you press, and

they never do what they would normally do in a program.  For instance,

in the sample session above, when you pressed the <CapsLock> key, the

CapsLock light did not toggle on the keyboard.  When you pressed <Ctrl>-

<Alt>-<Del>, the computer did not reboot.  SCANCODE is intercepting the

keys before they ever get to the BIOS - and the BIOS usually determines

what is SUPPOSED to happen when you press (or release) a particular key

on the keyboard.

Usually, when you press a key on the keyboard and hold it down, the

keyboard keeps sending "press" Scan Code(s) to the computer until you

release the key.  Only when you finally release the key does the

computer send a single release Scan Code (or set of Scan Codes).

SCANTEST "absorbs" all of the duplicated "press" Scan Codes, and, in

essence, hides them from you.  You will see a particular key press only

appear once in SCANTEST (when you first press the key), even though the

keyboard is actually continuing to send "press" Scan Codes the whole

time you have the key pressed.

NOTE:  The <Pause/Break> key is the exception to this rule.  It does not

continue to send "press" Scan Codes if you hold it down the way

the all the other keys do.  It sends a single set of "press" Scan

Codes, and then immediately sends a set of "release" Scan Codes,

and then stops, even if you keep holding the key down.  Try

holding down the <Pause/Break> key in SCANTEST to see what

happens, and compare this to what happens when you press and hold

down the other keys on the keyboard.

SCANTEST 1.00            Page 6 of 14

I purposely threw in a couple of "non-standard" keystrokes in this

example.  These are keystrokes that you don't often type yourself, and

when you do, the keyboard actually sends some WEIRD stuff to the

computer.  The BIOS inside your computer normally handles all of this

weirdness for you, and translates what comes from the keyboard into the

useful information you normally see on the screen.

I won't go into a lot of detail here about the unusual keystrokes,

because I go into GREAT detail about them in the documentation for

SCANCODE.  I will tell you, though, that any particular key on the

keyboard does not always send the same thing to the computer.  Sometimes

it changes, based on whether or not you have other keys pressed at the

same time, or even how many times you pressed another key before you

pressed this one.

The keys that sometimes "mutate" are <PrintScreen/SysReq>,

<Pause/Break>, and the ten "Grey" cursor-control keys (<GreyInsert>,

<GreyDelete>, <GreyHome>, <GreyEnd>, <GreyPageUp>, <GreyPageDown>,

<GreyUp>, <GreyDown>, <GreyLeft>, and <GreyRight>).  The keys that can

sometimes cause them to "mutate" are <NumLock> and the "Shift" keys

(<LeftShift>, <RightShift>, <LeftControl>, <RightControl>, <LeftAlt>,

and <RightAlt>, which is sometimes labeled <AltGr>).

One example of this "mutation" is shown in the sample session above,

when the <PrintScreen/SysReq> key was pressed.  The SCANTEST output

described what got sent to the computer as <FakeShift>-<PrintScreen>.

The keyboard literally sent a "fake" <LeftShift> key to the computer, to

"trick" it into thinking you pressed <Shift>-<PrintScreen> when you

really didn't.  The keyboard actually sends a lot of <FakeShift> keys to

the computer under certain circumstances, as you will discover if you

play around with SCANTEST very much.

Another example of this "mutation" is shown above, when the

<Pause/Break> and <Ctrl>-<Pause/Break> keystrokes were pressed.  When

the <Pause/Break> key was pressed all by itself, the keyboard sent a

<Pause> to the computer.  When the <Ctrl> and <Pause/Break> keys were

pressed at the same time, the keyboard sent a <Break> to the computer

(not a <Pause> like it did before).  The keyboard literally "mutated"

what was sent to the computer when you pressed the <Pause/Break> key,

based on whether or not you had the <Ctrl> key pressed at the same time.

These "mutations" are, in a demented sort of way, quite interesting.

They are, on the other hand, quite obnoxious and can cause you all sorts

of grief.  Again, I discuss this in annoyingly GREAT detail in the

documentation for SCANCODE, so I won't do it again here.  Read the

SCANCODE documentation if you want to know more, including a discussion

about why IBM thought "mutation" was a good idea.

SCANTEST 1.00            Page 7 of 14

WHAT ARE SCAN CODES?

��������������������

If you hit the <a> key on the keyboard, you normally see an "a" show up

on the screen.  What the keyboard sends to the computer is not really an

"a", though, but rather is simply a Scan Code that tells the computer

the physical location on the keyboard of which key was pressed (you can

think of it sort of like a row/column number).  It is the computer, not

the keyboard, that decides that the key you pressed is the "a" key, and

that it should put an "a" on the screen.

There are dozens of different keyboard layouts that are available from

different manufacturers, most of them for foreign countries.  The

standard United States keyboard is commonly called the QWERTY keyboard,

since these are the first several keys along the first alphabetic row of

the keyboard.  There is also another relatively common one for US users,

called the Dvorak keyboard.  On the Dvorak keyboard, where the <q> is on

a QWERTY keyboard isn't even a letter - it's the quote key (').  The

keyboard doesn't know, or even care, whether it is <a> or <'> or

whatever you want it to be.  All the keyboard knows is to send the

appropriate Scan Code to the computer when you press or release the key

(see the table below, on page 10, for the different Scan Codes that are

sent by the keyboard).

When you press (or release) a key on the keyboard, the keyboard

generates what is known as a hardware interrupt (also called an IRQ or

Interrupt ReQuest).  This tells the computer that the keyboard has a

keystroke waiting for it.  When the computer is ready for the keystroke,

it asks the keyboard to tell it which key was pressed or released.  The

keyboard responds by giving the computer a Scan Code.  Each key on the

keyboard has a unique Scan Code (or set of Scan Codes) when it is

pressed, and a different Scan Code (or set of Scan Codes) when it is

released.

The Scan Code is then (usually) processed by the BIOS (Basic Input

Output System).  Depending on which key was pressed or released, and

which key(s) were pressed or released before that, the BIOS either

stuffs an ASCII character into the keyboard buffer, remembers that the

key was a "Shift Key" (<Shift>, <Control>, <Alt>) or "Lock key"

(<CapsLock>, <NumLock>, <ScrollLock>), or performs some other function,

such as printing the screen in the case of <PrintScreen>, or rebooting

the computer in case of <Ctrl>-<Alt>-<Del>.

SCANTEST 1.00            Page 8 of 14

There is not a 1-to-1 correspondence between Scan Codes and ASCII

characters.  For example, the following sequence of keystrokes would

normally generate the ASCII character "A":

Push<Shift>, Push<a>, Release<a>, Release<Shift>

The following sequence of keystrokes would also generate an "A":

Push<CapsLock>, Release<CapsLock>, Push<a>, Release<a>,

Push<CapsLock>, Release<CapsLock>

There are probably at least half a dozen other ways to generate an ASCII

"A" also (although not ALL the different ways work in ALL programs).

To summarize, ASCII characters are NOT Scan Codes, and there is no

direct relationship between the two.  The keyboard sends Scan Codes to

the computer, and software "translates" the Scan Codes into the ASCII

characters that you see on the screen.  Exactly what your computer does

when you press (or release) a key is not (usually) decided by the

keyboard.  In particular, it is not decided by what is imprinted on the

plastic "keys" on the keyboard.  It is decided by the software inside

your computer.

SCANTEST 1.00            Page 9 of 14

DECIMAL KEYBOARD SCAN CODES (STANDARD US QWERTY KEYBOARD)

��������������������������������������������������������������������ͻ

�  �      �16� q     �32� d      �48� b      �64� F6     �80� Down   �

� 1� Esc  �17� w     �33� f      �49� n      �65� F7     �81� PgDn   �

� 2� 1    �18� e     �34� g      �50� m      �66� F8     �82� Ins    �

� 3� 2    �19� r     �35� h      �51� ,      �67� F9     �83� Del    �

� 4� 3    �20� t     �36� j      �52� .      �68� F10    �84� SysReq �

� 5� 4    �21� y     �37� k      �53� /      �69� NumLk  �85�        �

� 6� 5    �22� u     �38� l      �54� RShift �70� ScrlLk �86�        �

� 7� 6    �23� i     �39� ;      �55� Grey*  �71� Home   �87� F11    �

� 8� 7    �24� o     �40� '      �56� Alt    �72� Up     �88� F12    �

� 9� 8    �25� p     �41� `      �57� Space  �73� PgUp   �89�        �

�10� 9    �26� [     �42� LShift �58� CapsLk �74� Grey-  �90�        �

�11� 0    �27� ]     �43� \      �59� F1     �75� Left   �91�        �

�12� -    �28� Enter �44� z      �60� F2     �76� Grey5  �92�        �

�13� =    �29� Ctrl  �45� x      �61� F3     �77� Right  �93�        �

�14� BkSp �30� a     �46� c      �62� F4     �78� Grey+  �94�        �

�15� Tab  �31� s     �47� v      �63� F5     �79� End    �95�        �

��������������������������������������������������������������������ͼ

DECIMAL KEYBOARD SCAN CODES (ENHANCED KEYBOARD)

�������������������������������������������������������������������ͻ

�224-28       � GreyEnter  �224-71� GreyHome  �224-80  � GreyDown   �

�224-29       � RightCtrl  �224-72� GreyUp    �224-81  � GreyPgDn   �

�224-42-224-55� PrtScr     �224-73� GreyPgUp  �224-82  � GreyIns    �

�224-53       � Grey/      �224-75� GreyLeft  �224-83  � GreyDel    �

�224-56       � RightAlt   �224-77� GreyRight �(Alt)-84� Alt-SysReq �

�(Ctrl)-224-70� Ctrl-Break �224-79� GreyEnd   �225-29  � Pause      �

�������������������������������������������������������������������ͼ

DECIMAL KEYBOARD SCAN CODES (WINDOWS 95/NT KEYBOARD)

�������������������������������������������������������������ͻ

�224-91� LeftWindows �224-92� RightWindows �224-93� MouseMenu �

�������������������������������������������������������������ͼ

SCANTEST 1.00            Page 10 of 14

A WORD FROM THE SPONSOR

�����������������������

SCANTEST is a free program.  If you like it and use it, do something

nice for someone else in return.  I will accept niceties to myself in

the form of $, but that is not why I wrote SCANTEST.  I wrote it mostly

as an aid for myself to test and troubleshoot some of my other programs,

especially one called SCANCODE.

There are other utilities out there sort of like SCANTEST, and some even

claim to show you the Scan Codes as they come out of the keyboard.  But,

the main purpose of every one I've ever tested is to show you the ASCII

characters, not the Scan Codes.  A program that is intent on showing you

ASCII characters is unable to intercept certain keystrokes, like

<Ctrl>-<Alt>-<Del>, <Ctrl>-<PrtScr>, or even something "simple" like

<Pause>.  SCANTEST has no problem at all with ANY keystroke, no matter

what it is.

You can freely copy and distribute SCANTEST.COM, as long as it is

distributed along with this SCANTEST.DOC, and neither file has been

modified in any way. You cannot charge anyone in any way for SCANTEST,

except to recover your actual costs for disks and shipping (you cannot

charge for "handling").  You do need my permission to distribute

SCANTEST as a "companion" to some other program.

SCANTEST 1.00            Page 11 of 14

OTHER PROGRAMS

��������������

If you like SCANTEST (or even if you don't), I have written some other

programs you may find useful:

CLOCK

Shows a clock on your screen all the time.  Also has two alarms

that beep at you when it's time for an appointment.

JOYKEYS

Lets you use your joystick(s) with any program.

MOUSKEYS

Lets you use your mouse with any program.

PRTSCR

Sends all of your PrintScreens to a file instead of a printer.  A

good way to transfer data between two programs, even if they won't

normally "talk" to each other.

PRTSCRFF

Modifies your PrintScreen so that each new PrintScreen gets put on

a new sheet of paper (especially useful if you have a Laser

printer).

SCANCODE

Type keystrokes automatically in the middle of other programs.

Helpful in getting past the "starting screens" of most programs, or

to create macros.  Can also be used to remap the keyboard.

SERIAL

Shows the status of your serial (COM) ports on screen.  Shows the

bit rate, parity, status of the leads (DTR, DSR, etc.).  An

excellent way to troubleshoot modem problems.

SLOWDOWN

Slows down a fast computer so you can run older programs that

require a PC/XT or PC/AT.

SCANTEST 1.00            Page 12 of 14

WARRANTY

��������

There is no warranty of any kind, either expressed or implied, supplied

with SCANTEST.  As with any software, it may or may not work with other

software or hardware that you may have.  I have done my best to make

sure SCANTEST won't screw anything up, but, again, there is no

guarantee.

If you find any "bugs" in this program, or see something that you wish

were different, please let me know.  I can only improve SCANTEST if I

know what you think needs to improve.

The source code for SCANTEST is available, for free, if you want it.  I

don't distribute the source code with the program, though, to keep the

size of the download small, and also because I know most people don't

care about the source code, anyway.  The source code is available from

my home page, http://members.aol.com/bretjohn.

HAVE FUN!

Bret Johnson

6775 South Logan Street

Littleton, CO  80122-1249

(303) 795-5084

bretjohn@juno.com  or

bretjohn@aol.com

I intend to always keep the latest versions of all of my programs on my

own web page.  The URL is:

http://members.aol.com/bretjohn

Keep looking there for updates to my programs.

SCANTEST 1.00            Page 13 of 14

REVISION HISTORY

����������������

1999-04-23  v1.00

SCANTEST 1.00            Page 14 of 14